Exam 1 Study Guide

Boolean Algebra

-Know DeMorgan’s law

-Be able to do truth tables and KMaps to minimize functions

-Sum of products form and minterms

Gates

-Be able to recognize different gates

-Make AND, OR, NOT gates with NAND & NOT gates

-Know what a block diagram is

Combinational Circuits

-Be familiar with full adder, multiplexers & decoders

-Be able to draw an adder, a decoder, and a multiplexer (2 to 3 bits) and label it correctly

-When wiring gets rough, label inputs, outputs, and gates

Flip-Flops

-Know their behavior, state tables

-S-R flip flop – know the 4 row table that implies you know the different states

-Familiar with counters and RAM circuits

-Know what they look like

-Know their characteristic tables and how they work

SR(JK) Flip Flop

| S | R | Q | Q+1 |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | Undefined (1 in JK) |
| 1 | 1 | 1 | Undefined (0 in JK) |

SR Short-hand

| S | R | Q(t+1) |
| --- | --- | --- |
| 0 | 0 | Qt (no change) |
| 0 | 1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | Undefined |

JK Flip Flop

| J | K | Q(t+1) |
| --- | --- | --- |
| 0 | 0 | Qt (no change) |
| 0 | 1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | Qt’ (toggle) |

D Flip Flop

| D | Q |
| --- | --- |
| 1 | 1 |
| 0 | 0 |

T Flip Flop

| T | Q |
| --- | --- |
| 1 | 0 |
| 0 | 1 |

Number and Base conversion

-Be able to convert between bases, 10, binary, hex, and octal

-No calculators allowed

-If going from decimal to hex, know all of the digits (0-F)

-Hex to octal or octal to hex: CONVERT TO BINARY FIRST

-BE PREPARED FOR A REALLY LONG NUMBER

-Radix point and fractional conversions (will be 1 question about this)

-Know the powers of 2 reciprocals, know at least out to 6 digits

-135.65213

2^7 = 128 (135-128=7) 1

2^6  0

2^5  0

2^4  0

2^3  0

2^2 = 4 (7-4 = 3) 1

2^1 = 2 (3-2=1)  1

2^0 = 1 (1-1=0  1

Binary  1000011

After radix point:

.65123 * 2 = 1.30246

1.30246 * 2 = 0.60492

0.640492 * 2 = 1.2984

1.2984 * 2 = 0.41968

0.41968 * 2 = 0.83936

Converts to .10100…

Integer representation

-Unsigned

-Signed

-1s compliment

-2s compliment

-Test will tell us which format is being used, know how what they mean

Character Codes

-BCD- 6-bit

-EBSCDIC- 8-bit

-ASCII- 7 or 8 bit

-Unicode- 16 bit

Exam is 4 pages, about 25 questions total

-KMaps

EXTRA CREDIT SOLUTION

SEG1 = T1+T3

SEG2 = T0+T1+T2+T3

SEG3 = T0+T1

SEG4 = T0+T1+T3

SEG5 = T0+T1+T2+T3

SEG6 = T2+T3

SEG7 = T0+T1+T3

Label the gates to show what segment they will be lighting up
	Draw the block diagrams

Clock  Counter  J1 & J2  T0, T1, T2, T3

Extra credit is from the T AND gates to the OR gates of the segments